IC TRAINING CENTER VIET NAM

FUNDAMENTAL IC DESIGN AND VERIFICATION COURSE

FINAL PROJECT

TIMER IP DESIGN SPECIFICATION

Student: Nguyễn Tân Thành

Class: IC30

Instructor: Nguyễn Thanh Trí

Contents

LIST OF TABLES	3

LIST OF FIGURES	3

1.Overview	4

1.1. Introduction	4

1.2. Main features	4

1.3. Block diagram	5

1.4. Interface signals	6

2.Register Specification	6

2.1. Register Summary 	6

2.2. Timer Control Register (TCR)	7

2.3. Timer Data Register 0 (TDR0)	8

2.4. Timer Data Register 1 (TDR1)	8

2.5. Timer Compare Register 0 (TCMP0)	9

2.6. Timer Compare Register 1 (TCMP1)	9

2.7. Timer Interrupt Enable Register (TIER)	10

3. Functional Description	12

3.1. APB slave	12

3.2. Diagram Explanation	13

4. History	16

# LIST OF TABLES

Table 1 : Interface signals of timer IP	6

Table 2 : Register Summary	6

Table 3 . Timer Control Register (TCR)	7

Table 4 : Timer Data Register 0 (TDR0)	8

Table 5 : Timer Data Register 1 (TDR1)	9

Table 6 : Timer Compare Register 0 (TCMP0)	9

Table 7 : Timer Compare Register 1 (TCMP1)	10

Table 8 : Timer Interrupt Enable Register (TIER)	10

Table 9 : Timer Interrupt Status Register (TISR)	11

# LIST OF FIGURES

Figure 1 : Block diagram of Timer IP	5

Figure 2 : Block diagram of counter	5

Figure 3 : Waveform of operation	12

Figure 4 : Sub block internal_cnt	13

Figure 5 : Sub block cnt_control	14

Figure 6 : Block diagram of TCR	14

Figure 7 : Block diagram of TDR0	15

Figure 8 : Block diagram of TDR1	15

Figure 9 : Block diagram of TCMP0	15

Figure 10 : Block diagram of TCMP1	16

Figure 11 : Block diagrams of TIER and TISR	16

# 1.Overview

## 1.1. Introduction

The timer is a fundamental and indispensable peripheral module integrated into modern microcontroller units (MCUs) and system-on-chips (SoCs). Its primary function is to provide a reliable and precise mechanism for generating specific timing intervals, thereby orchestrating the timing of various internal operations and external events. By operating from the system clock or an independent oscillator, it delivers a critical timing reference that allows the central processing unit (CPU) to execute tasks asynchronously or at predetermined rates, significantly enhancing system efficiency and control.

The utility of timer modules extends across a vast array of critical applications within embedded systems and digital circuits. They are primarily employed for the generation of precise digital waveforms, including pulses and Pulse Width Modulation (PWM) signals, which are essential for controlling motors, LEDs, and power converters. Furthermore, timers are crucial for creating programmed delays, scheduling periodic events, and generating interrupt requests to signal the CPU that a specific time-based condition has been met. This versatility makes them a cornerstone for implementing real-time operating system (RTOS) schedulers, communication protocol timeouts, and sensor data sampling routines.

## 1.2. Main features

This timer module is architected around a 64-bit count-up counter and features a 12-bit address space for register access. It is configured as an APB slave, with its register set programmable via the APB bus. The IP supports a standard compliance level, which includes 32-bit APB transfers with no wait states and no error handling, and an advanced level that adds support for single-cycle wait states, error handling, byte access, and debug-mode halt functionality. The module operates from a 200 MHz system clock and utilizes an active-low asynchronous reset. The counter can be clocked directly from the system clock or from a divided version, with a division factor of up to 256. Additionally, the timer provides a configurable interrupt output that can be enabled or disabled as required.

## 1.3. Block diagram

Figure 1: Block diagram of Timer IP

Figure 2: Block diagram of counter

## 1.4. Interface signals

Table 1: Interface signals of timer IP

| Signal name | Width | Direction | Description |
| --- | --- | --- | --- |
| sys_clk | 1 | input | System's clock, sampling at the edge of clk |
| sys_rst_n | 1 | input | Low active reset, release for normal operation |
| tim_psel | 1 | input | Select which slaves need to be in charge during the transaction |
| tim_pwrite | 1 | input | Signal indicating whether write or not |
| tim_penable | 1 | input | Signal indicating the start of Access phase |
| tim_paddr | 12 | input | Select which addr in slave required for execution |
| tim_pwdata | 32 | input | Data that will be written to slave from master |
| tim_prdata | 32 | output | Data that master received from slave |
| tim_pstrb | 4 | input | Signal enables sparse data transfer |
| tim_pready | 1 | output | Signal indicating that slave is ready for the transfer |
| tim_pslverr | 1 | output | Signal indicating an error condition during APB transfer |
| tim_int | 1 | output | Update the status of interrupt reg, check the trigger condition |
| dbg_mode | 1 | input | dbg_mode does not change after timer_en is High |

# 2.Register Specification

## 2.1. Register Summary

Table 2: Register Summary

| Offset | Abbreviation | Register name |
| --- | --- | --- |
| 0x00 | TCR | Timer Control Register |
| 0x04 | TDR0 | Timer Dara Register 0 |
| 0x08 | TDR1 | Timer Dara Register 1 |
| 0x0C | TCMP0 | Timer Compare Register 0 |
| 0x10 | TCMP1 | Timer Compare Register 1 |
| 0x14 | TIER | Timer Interrupt Enable Register |
| 0x18 | TISR | Timer Interrupt Status Register |
| 0x1C | THCSR | Timer Halt Control Status Register |
| Others | Reserved |  |

## 2.2. Timer Control Register (TCR)

Offset address: 0x0

Reset value: 0x0000_0100

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  | DIV_VAL[3:0] |  |  |  | Reserved |  |  |  |  |  | DIV_ EN | TIM_ EN |
|  |  |  |  | rw |  |  |  |  |  |  |  |  |  | rw | rw |

Table 3. Timer Control Register (TCR)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:12 | Reserved | - | 20’b0 | Reserved |
| 11:8 | DIV_VAL[3:0] | RW | 4’b0001 | Counter control mode setting: • 4’b0000: Counting speed is not divided • 4’b0001: Counting speed is divided by 2 (default) • 4’b0010: Counting speed is divided by 4 • 4’b0011: Counting speed is divided by 8 • 4’b0100: Counting speed is divided by 16 • 4’b0101: Counting speed is divided by 32 • 4’b0110: Counting speed is divided by 64 • 4’b0111: Counting speed is divided by 128 • 4’b1000: Counting speed is divided by 256 • Others: reserved When setting the prohibit value, div_val is not changed. Note: user must not change div_en while timer_en is High |
| 7:2 | Reserved | RO | 6’b0 | Reserved |
| 1 | DIV_EN | RW | 1’b0 | Counter control mode enable. • 0: Disabled. Counter counts with normal speed based on system clock • 1: Enabled. The couting speed of counter is controlled based on div_valNote: user must not change div_en while timer_en is High |
| 0 | TIM_EN | RW | 1’b0 | Timer enable • 0: Disabled. Counter does not count. • 1: Enabled. Counter starts counting |

## 2.3. Timer Data Register 0 (TDR0)

Offset address: 0x4

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Timer Data Register 0 (TDR0)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TDR0 | RW | 32’h0000_0000 | Lower 32-bit of 64-bit counter |

## 2.4. Timer Data Register 1 (TDR1)

Offset address: 0x8

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5: Timer Data Register 1 (TDR1)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TDR1 | RW | 32’h0000_0000 | Upper 32-bit of 64-bit counter |

## 2.5. Timer Compare Register 0 (TCMP0)

Offset address: 0xC

Reset value: 0xFFFF_FFFF

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCMP0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6: Timer Compare Register 0 (TCMP0)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TCMP0 | RW | 32’hffff_ffff | Lower 32-bit of 64-bit compare value |

## 2.6. Timer Compare Register 1 (TCMP1)

Offset address: 0x10

Reset value: 0xFFFF_FFFF

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TCMP1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7: Timer Compare Register 1 (TCMP1)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TCMP1 | RW | 32’hffff_ffff | Upper 32-bit of 64-bit compare value |

## 2.7. Timer Interrupt Enable Register (TIER)

Offset address: 0x14

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  | int_en |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  | rw |  |

Table 8: Timer Interrupt Enable Register (TIER)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:1 | Reserved | RO | 31’h0 | Reserved |
| 0 | int_en | RW | 1’b0 | Timer interrupt enable 0: Timer interrupt is disabled. 1: Timer interrupt is enabled. When this bit is 0, no timer interrupt is output. When this bit is 1, timer interrupt can be output when reaching trigger condition. Clearing this bit to 0 while interrupt is asserting will mask the interrupt to 0 but does not affect the interrupt pending bit TISR.int_st bit. |

2.8. Timer Interrupt Status Register (TISR)

Offset address: 0x18

Reset value: 0x0000_0000

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  | int_st |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  | rw1c |  |

Table 9: Timer Interrupt Status Register (TISR)

| Bit | Name | Type | Default value | Description |
| --- | --- | --- | --- | --- |
| 31:1 | Reserved | RO | 31’h0 | Reserved |
| 0 | int_st | RW1C | 1’b0 | Timer interrupt trigger condition status bit (interrupt pending bit) 0: the interrupt trigger condition does not occur. 1: the interrupt trigger condition occurred. Write 1 when this bit is 1 to clear it Write 0 when this bit is 1 has no effect Write to this bit when it is 0 has no effect. Note: When interrupt trigger condition occurred (counter reached compare value), counter continues to count normally. |

# 3. Functional Description

## 3.1. APB slave

The timer IP has an APB slave to receive settings from user. This APB slave has below features:

- 12-bit width address

- 32-bit width read/write data

- Not support wait state and error response

<Waveform Figure>

Figure 3: Waveform of operation

At the 4th clock, the transfer to write 32’h1 to TCR is set up and enables the bit timer_en to toggle up in the next 2 cycles (6th edge). Then, the counter starts to operate in the normal mode and TDR0, TDR1 stores the values of 64 bits of counter.

At the 7th clock, 32’h2 is written into the TCMP0 register and the same transfer of 32’h1 is executed in TCMP1 register (at the 10th clock).

At the clock, int_en rises to the high level which is illustrated by writing 32’h1 into TIER, when TDR0 reaches TCMP0 and TDR1 is as same as TCMP1 at 19th clock, int_st will be toggled up and result in HIGH output tim_int.

## 3.2. Diagram Explanation

The “Counter” block contains 2 more sub-blocks, “internal_cnt” and “cnt_control”. Both of them are configured for correct operation during the control mode of counter. Regarding the “internal_cnt” block, it enables a variable “i” to count up and allows the counter to update its value until “i” reaches a specific value. Finally, the “cnt_control” will update the data based on the given “i” from the left block. The detailed diagrams of these sub-blocks are given below:

Figure 4: Sub block internal_cnt

The above sub-block can operate to count in parallel with the sub-block “Cnt_control” below so that it can release an internal output “i” to control the “count [63:0]”. The role of this block is at the temporary value: , when i reaches this value, the counter can be allowed to updata the new value.

Figure 5: Sub block cnt_control

Sub-block “cnt_control” can execute the counting performance of “count [63:0]” in accordance with “internal_cnt” block’s allowance. With regards to 2 inputs “wr_DR0” and “wr_DR1”, both of them are used to update the counter immediately with the value written into TDR0 and TDR1.

Figure 6: Block diagram of TCR

In the TCR block, the two multiplexers at the left-hand side can select the valid value for div_val [3:0] based on wdata[11:8] and div_en (passed to TCR[1] via wdata[1]). Then, there are two ranges in the bit width of TCR (TCR[11:8] and TCR[1:0]) that can be updated based on the wr_en signal input.

Figure 7: Block diagram of TDR0

Figure 8: Block diagram of TDR1

Figure 9: Block diagram of TCMP0

Figure 10: Block diagram of TCMP1

Figure 11: Block diagrams of TIER and TISR

Regarding the TISR block, its function is to rise the int_st up when there is a matching detection between count [63:0] and {TCMP1, TCMP0}. Additionally, it also has a function to write 1 to clear the its high output, which can be detailed described at the left-hand side of the figure.

# 4. History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
|  | 1.0 | Newly created | Nguyễn Tân Thành |